Safety Data Sheet

Revision Date: 2025-11-10

- Product and company identification

Product Name: Calmabon Liquid.

Identification: Chlorine-free special fertilizer.

Recommended use: Fertilizer, only use as directed.

Details of Supplier:

Hygrotech Properties (Pty) Ltd, 1 Gerard Braak Street, Pyramid, Pretoria, 0120.

Emergency Telephone numbers:

Griffon Poison Information Centre: 082 446 8946.

Hygrotech Head Office: 012 545 8000 (o/h).

- Hazard(s) identification

Classification of the substance or mixture

This product is classified as hazardous according to the criteria in South Africa - GHS classification and labelling of chemicals – SANS10234 and the Regulations for Hazardous Chemical Agents - 2021.

GHS Classification

| Hazard Class | Category | Hazard Statement Number |
| --- | --- | --- |
| Serious Eye Damage/Irritation | 1 | H318 |

Label Elements

Pictograms:

Signal Word:

Danger

Hazard Statements:

| Statement Number | Hazard Statement |
| --- | --- |
| H318 | Causes serious eye damage. |

Precautionary Statements:

General -

| Statement Number | Precautionary Statement |
| --- | --- |
| P101 | If medical advice is needed, have product container or label at hand. |
| P102 | Keep out of reach of children. |
| P103 | Read carefully and follow all instructions. |

Prevention -

| Statement Number | Precautionary Statement |
| --- | --- |
| P280 | Wear protective gloves, protective clothing, eye and face protection. |
| P264 + P265 | Wash hands and face thoroughly after handling. Do not touch eyes. |

Response -

| Statement Number | Precautionary Statement |
| --- | --- |
| P317 | Get medical help. |
| P305 + P354 + P338 | IF IN EYES: Immediately rinse with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing. |

Other Hazards

None known for the product.

- Composition/information on ingredients

Substance/mixture

Mixture.

Ingredients with Hazard Concerns (GHS)

According to UN GHS criteria.

| Hazardous Components – Chemical Name | CAS Number | Weight - % | GHS Classification |
| --- | --- | --- | --- |
| Nitric acid, ammonium calcium salt | 15245-12-2 | 10 – 20% | Acute Toxicity Oral, Category 4. Serious eye Damage/Irritation, Category 1. |
| Disodium octaborate tetrahydrate | 12280-03-4 | <0.5% | Reproductive Toxicity, Category 1B. |

NOTE: The other ingredients do not cause or contrinute toward the correct GHS classification of Calmabon Liquid and are therefore, in terms of the South African Regulations for Hazardous Chemical Agents - 2021; Regulation 14(b), not listed in the table above.

- First Aid measures

Description of First Aid measures

| General advice | In case of treatment resulting from excessive exposure, provide this SDS and product label to medical personnel. Emergency personnel should wear protective clothing appropriate to the type and degree of contamination. First Aid personnel should pay attention to their own safety. Immediately remove contaminated clothing and move the affected person away from the contamination area. Keep the person warm, calm, and covered up. |
| --- | --- |
| Inhalation | Allow the patient to rest in a well-ventilated area. If breathing is difficult, have qualified personnel administer oxygen. If breathing has stopped, use artificial respiration to support vital functions. Remove and isolate contaminated clothing and shoes. Make sure medical personnel are aware of the substance involved and take precaution to protect them. Obtain medical attention if concerned or unwell. |
| Ingestion | If this product is swallowed, immediately call a poison control centre or medical practitioner for treatment advice. If conscious, rinse mouth thoroughly with water. Never give anything by mouth to an unconscious or convulsing person. Do not induce vomiting unless directed to do so by a medical professional. Have the patient lean forward with head down to avoid breathing in of vomits if spontaneous vomiting occurs. |
| Skin contact | Remove all contaminated clothing and shoes. Decontaminate the affected skin area with running water under the safety shower. Minimum recommended flushing time is 15 – 20 minutes, especially if an adverse skin reaction occurs. Obtain medical attention if irritation occurs or persist. Contaminated clothing should be washed prior to re-use. |
| Eye contact | Immediately rinse/flush the eyes gently with running water for at least 15 minutes keeping the eyelids apart. Cold water must be used. Check for and remove contact lenses if easy to do so. Continue with the rinsing. Prevent rubbing of the eyes. Obtain immediate medical attention. |

Most important symptoms/effects, acute and delayed

Skin – may cause mild temporary skin irritation after repeated exposure in sensitive individuals. There may be 	irritation and redness at the site of contact.

Eye – corrosive to eyes and could causes serious eye damage.

Ingestion – Could cause irritation of the gastric/intestinal mucosa resulting in vomiting, diarrhoea, nausea, and weakness if ingested in large volumes.

Inhalation – no adverse effects known for the product.

Indication of any immediate medical attention and special treatment needed

Notes to physician:

No specific antidote required. Treat patient symptomatically and supportively.

- Fire-fighting measures

| Suitable (and unsuitable) extinguishing media | Small fires – use water spray, carbon dioxide, foam, or dry chemicals. Larger fires – use foam, water fog or water spray. Fire water must be contained for safe disposal. Do not use high volume water jets due to potential contamination. |
| --- | --- |
| Specific hazards arising from the chemical including thermal decomposition products | The product is not flammable. When involved in a fire and exposed to extremely high temperatures, the components of this product may decompose to produce irritating vapours and hazardous compounds including oxides of nitrogen (NOx), carbon (CO and CO2), calcium, and magnesium as well as ammonia. |
| Special protective equipment | Fire responders should wear emergency equipment including positive pressure self-contained breathing apparatus with a full-face mask. If possible, prevent runoff water from entering storm drain bodies of water or other environmentally sensitive areas. |
| Additional measures | Fight the fire from a maximum distance and keep containers cool by spraying with water. Act in accordance with the site’s Internal Emergency Plan and the procedures for actions to be taken during and after an accident or other emergencies. |

- Accidental release measures

| Personal precautions, protective equipment, and emergency procedures | In case of a release, clear the affected area and protect people. Appropriately trained personnel dressed in the prescribed personal protective equipment should respond to uncontrolled releases. Emergencies should be handled in accordance with the facility’s pre-planned procedures. Contain the spill if it can be done without risk and clean-up immediately. Avoid sources of ignition in the spill area Ventilate the area of the spill or leak, especially when in confined areas and do not breathe in spray/vapours. Avoid contact of spilt material with eyes, skin and clothes and do not touch or walk through spilled material. Wear appropriate protective clothing recommended in Section 8 of the SDS. |
| --- | --- |
| Environmental precaution | Prevent spillage or further leakage (if safe to so) and the contamination of environmentally sensitive areas. Do not allow the spilt product to enter water courses and drains and avoid contact with soil. Report spills and releases as required to appropriate authorities if the spilt product has caused environmental pollution (sewers, water ways, or soil). |
| Containment and clean-up | For small spills – clean up using gloves, safety glasses and appropriate body protection. Soak up the spilt product with suitable inert absorbent material from the spill kit. Apply enough absorbent material to completely cover the spilt product. Shovel up the spilt material and place into a labelled waste container. Label and cover the container for subsequent safe disposal. Dispose of collected spilt material as hazardous waste to an incineration facility if an available option. For large spills - personal protective equipment should be level C: triple gloves, chemical resistant suit, and air purifying respirator with a high-efficiency particulate filter. Contain/dyke/cover to prevent dispersal using absorbent socks, pillows or pads supplied in a spill kit. Collect the spilt product and place it into suitable labelled containers for proper disposal as hazardous waste. |
| Reference to other SDS sections | See Section 1 for emergency contact information. See Section 8 for information on appropriate personal protective equipment. See Section 13 for additional waste treatment information. |

- Handling and storage

| Precautions for safe handling | Wear the protective clothing as indicated in Section 8 of this SDS. Prevent personal exposure to the product – prevent skin and eye contact. Wash hands and face thoroughly after handling the product and do not eat, drink, smoke or apply cosmetics while handling. Always use in a well- ventilated work area. Locate emergency showers and eye-rinsing facility near the work/handling area. Maintain good normal industrial hygiene and housekeeping practices in areas where the product is used/handled. Remove contaminated clothing immediately if the product gets inside. Contaminated work clothing should not be allowed out of the workplace. Regular cleaning of work area and work clothing is recommended. Keep unprotected persons away from the area where the product is being applied. |
| --- | --- |
| Conditions for safe storage, including any incompatibilities | The entrance to storage facilities should be granted only to appropriately trained personnel. Keep out of reach of children, uninformed persons, and animals. Containers must always be clearly labelled and tightly closed/sealed when not in use. Store containers in a cool dry location, away from direct sunlight and sources of intense heat, or where freezing is possible. Store away from incompatible materials like combustible materials, reducing agents, organic compounds, and strong acids. Inspect all incoming containers before storage to ensure that containers are properly labelled and are not damaged. Empty containers should be handled with care. It is recommended to have appropriate spill control kits equipped with clean-up tools near storage areas (see Section 6). Store in accordance with national and local regulations. |

- Exposure controls/personal protection

## Components with workplace control parameters – National Occupational Exposure Limits

## Occupational exposure limits have not been established for any of the product’s ingredients by the South African Department of Employment and Labour.

| Appropriate engineering controls | Use with adequate general or mechanical exhaust ventilation to prevent inhalation of spray or mist and to maintain airborne concentration as low as possible. Eyewash stations and safety showers should be near areas where this product is used or sprayed. |
| --- | --- |
| Individual protection measures | Hand: impervious chemical resistant gloves recommended for hand protection (e.g., butyl rubber, nitrile rubber, etc.). The gloves should be replaced immediately in case of damage or signs of wear. Eye/Face: safety eyewear compliant with an approved standard should be used when a risk assessment indicates this is necessary to avoid eye exposure to liquids and splashes. Safety goggles together with a face shield are recommended when a respirator is not used. Skin: impervious overalls, apron, shoes, and socks as required to prevent skin contact and contamination of personal clothing. Overalls must be buttoned to the neck and sleeves worn over the gloves. Respiratory: protection selection must be based on known or anticipated exposure levels, the hazards of the product and the safe working limits of the selected respiratory equipment. In operations where the risk assessment indicates that there could be exposure e.g., when exposure to fumes/vapours/spray mist is expected, when handling in an enclosed area or when prolonged exposure is possible, an approved respirator (full face mask) with a particulate filter and an organic vapour cartridge or supplied air respirator should be used. For emergency conditions, use an approved positive-pressure self-contained breathing apparatus. |
| Environmental exposure controls | In accordance with the national and local legislation for the protection of the environment, it is recommended to avoid environmental spillage or releases of both the product and its container. |

- Physical and chemical properties

| Phase | Liquid |
| --- | --- |
| Colour | Milky - white |
| Odour | Odourless |
| pH (1%) | 3-5 |
| Specific gravity (20°C) | 1.5 |
| Solubility | Soluble in water |
| Octanol/water partition coefficient: | No data available |
| Flammability | Not flammable |
| Oxidizing properties | Not an oxidizer |
| Boiling point (°C) | >100 |
| Decomposition temperature | No data available |

Other Hazard Information

None known for the product.

- Stability and reactivity

| Reactivity | The product is not reactive under normal ambient and anticipated storage and handling conditions of temperature and pressure. |
| --- | --- |
| Chemical stability | Hazardous polymerization will not occur. Stable under normal ambient conditions of use, storage, and transport. |
| Conditions to avoid | Storage without good ventilation and exposing to excessive heat. Avoid contact with strong bases – may release ammonia. Avoid contact with strong oxidizing agents – may decompose. |
| Incompatible materials | Combustible materials, reducing agents, organic compounds, and strong acids. |
| Hazardous decomposition products | Does not decompose when used for intended uses. Could decompose under fire or during burning and at high temperatures releasing oxides of nitrogen (NOx), carbon (CO and CO2), calcium, and magnesium as well as ammonia. |

- Toxicological information

Information on likely routes of exposure

The product can be absorbed into the body by inhalation of spray mist and by skin/eye contact.

Assessment of acute toxicity:

Experimental toxicological test data are not available for the product. Assessment of acute toxicity is based on the ingredients.

Acute Toxicity Oral: The product is not classified.

Acute Toxicity Dermal: The product is not classified.

Acute Toxicity inhalation: The product is not classified.

| Product/ingredient Name | Dose Acute - | Species | Test Result |
| --- | --- | --- | --- |
| Calmabon Liquid | 3 385 mg/kg | Rat | ATE(MIX) Oral |
| Nitric acid, ammonium calcium salt | 300 - 2 000 mg/kg bw | Rat | LD50 Oral |
| Nitric acid, ammonium calcium salt | > 2 000 mg/kg | Rat | LD50 Dermal |

| Irritation Dermal/Skin and Eyes | Assessment of irritation effects (skin/eyes): Based on available data, the classification criteria are not met. |
| --- | --- |
| Skin Corrosion/Eye Damage | Assessment of skin corrosion/eye damage: Based on available data, the classification criteria met for serious eye damage. Nitric acid, ammonium calcium salt: Experimental animal - rabbit. Instillation of the ingredient into rabbit eyes resulted in effects on the cornea, iris, and conjunctivae. The effects persistence up to the end of the observation time of 21 days. Irreversible effects. Data source: ECHA. |
| Respiratory/Skin Sensitization | Assessment of sensitization: Based on available data, the classification criteria are not met. |
| Germ Cell Mutagenicity | Assessment of mutagenicity: Based on available data, the classification criteria are not met. |
| Carcinogenicity | Assessment of carcinogenicity: Based on available data, the classification criteria are not met. |
| Reproductive Toxicity and Developmental Toxicity | Assessment of reproduction toxicity: Based on available data, the classification criteria are not met. Disodium octaborate tetrahydrate - Animal feeding studies in rats, mice, and dogs at high doses have demonstrated effects on fertility and the testes. Studies in rats, mice, and rabbits at high doses have demonstrated developmental effects on the foetus, including foetal weight loss and minor skeletal variations. |
| Specific Target Organ Toxicity (Single Exposure) | Assessment of STOT (single): Based on available data, the classification criteria are not met. |
| Specific Target Organ Toxicity (Repeated Exposure) | Assessment of repeated dose toxicity: Based on available data, the classification criteria are not met. |
| Aspiration Hazard | Assessment of repeated dose toxicity: Based on available data, the classification criteria are not met. |

Delayed and immediate effects as well as chronic effects from short and long-term exposure

None known for the product.

- Ecological information

Ecotoxicity

The product is of low toxicity to aquatic systems.

The product will not enter the environment under normal use if used as indicated on the label. Care should however be taken to avoid any additional release, for example through inappropriate disposal.

No eco-toxicological data available for the formulated product. The assessment of aquatic toxicity is based on the ingredient, nitric acid ammonium calcium salt.

| Nitric acid, ammonium calcium salt (data source – ECHA) |  |  |  |
| --- | --- | --- | --- |
| Measurement | Acute Toxicity Result and Exposure (h) | Species | Genus |
| LC50 | 447 mg/L (48h) | Cyprinus Carpio (carp) | Fish |
| EC50 | >100 mg/L (48h) | Daphnia magna | Aquatic Invertebrates |
| EC50 | >100 mg/L (72h) | Pseudokirchneriella subcapitata | Algae |

Persistence and Degradability

Stability

In aqueous solution, nitric acid, ammonium calcium salt is completely dissociated into the calcium ion (Ca2+), the ammonium ion (NH4+) and the nitrate anion (NO3-) which cannot be further degraded.

Hydrolysis of nitric acid, ammonium calcium salt does not occur.

Nitric acid, ammonium calcium salt is an inorganic salt, soluble in water. It is a neutral salt; the ions have little tendency to react with water. Hydrolysis is therefore not a relevant parameter for this substance and testing does not appear scientifically necessary.

Biodegradation

Readily biodegradation study does not need to be conducted since the substance is inorganic.

Mobility in Soil

Nitrate is not bound to the soil and will follow water movements. Nitrate can therefore leach when the soil receives more water than it can take up. Calmabadon Liquid is also soluble in water and is expected to be leachable through normal soil.

Bioaccumulative Potential

Due to the high aqueous solubility of nitric acid ammonium calcium salt, it will exist in a dissociated form in an aqueous solution. It therefore has a low potential for bioaccumulation.

Other Environmental and Adverse Effects:

No specific data available for the product.

- Disposal information

| Waste handling and disposal methods | Avoid and minimize the generation of waste. Product related waste: Dispose of product related waste in according to the requirements of local/regional/national/international regulations. Do not allow to enter groundwater, surface water or drains and do not contaminate water, food, or feed by storage or disposal. Rinse empty container 3 times with a volume of water equal to one tenth of the volume of the container and add to the spray tank. Dispose as hazardous waste by thermal incineration (if an available option) or to a licensed waste-handling facility. |
| --- | --- |
| Disposal of packaging | Container disposal: Empty containers must not be disposed of as part of general waste. Non-refillable container and should not be reused or refilled. Triple rinse container (or equivalent) promptly after emptying. Empty the rinsate into the application equipment or a mix tank or store rinsate for later use or disposal. Puncture and flatten (where appropriate) the nominally empty container. Offer for recycling by taking the container to a collection point from where it can be collected by an approved service provider. Dispose of to a hazardous waste facility if recycling is not an option. |

- Transport information

The product is not classified for the transport of Dangerous Goods purposes.

- Regulatory information

Safety, health, and environmental regulations specific for the product in question

| Hazard symbol |  |
| --- | --- |
| Classification | C: Corrosive. |
| Risk phrases | R41: Risk of serious damage to eyes. |

Key to Abbreviations

| AND | European Provisions concerning the International Carraige od Dangerous Goods by inland Waterways |
| --- | --- |
| ADR | The European Agreement concerning the International Carraige of Dangerous Goods by Road |
| ATE | Acute Toxicity Estimate |
| CAS Number | Chemical Abstracts Service Number |
| COD | Chemical Oxygen Demand |
| ECHA | European Chemicals Agency |
| GHS | Globally Harmonised System of Classification and Labelling of Chemicals |
| IATA | International Air Transport Association |
| ICAO | International Civil Aviation Organisation |
| IMDG | International Maritime Dangerous Goods |
| LogPow | Logarithm of the octanol/water partition coefficient |
| LD50 | Lethal Dose 50 |
| LC50 | Lethal Concentration 50 |
| RID | The Regulations concerning the International Carraige of Dangerous Goods by Rail |
| SDS | Safety Data Sheet |
| STOT | SpecificTarget Organ Toxicity |
| TWA | Time Weighted Average |
| UN | United Nations |

No known specific country national and/or local regulations applicable to the product (including its ingredients). A summary of South African specific general laws/regulations are supplied below.

Country Specific Legal Requirements

| Country | Requirements |
| --- | --- |
| South Africa | Registration Requirements: Fertilizer, Farm Feeds, Agricultural Remedies and Stock Remedies Act, 1947 (Act 36 of 1947). Pesticide Handling, Storage and Disposal Safety: SANS10206: 2020. Safety Data Sheet and Occupational Exposure Limit Requirements: Regulations for Hazardous Chemical Agents – 2021 – SA Occupational Health and Safety Act. SANS11014:2010 Safety Data Sheet for Chemical Products – Content and Order of Sections. Control of and handling of poisonous/hazardous and non-poisonous/non-hazardous substances/chemicals in workplaces: Hazardous Substances Act, 1973 (Act No.15 of 1973). Occupational Health and Safety Act No. 85 of 1993. Extended Producer Responsibility: National Environmental Management: Waste Act, 2008 (Act No.59 of 2008) NEMWA: Extended Producer Responsibility Scheme for the Pesticide Sector. Department of Environment, Forestry and Fisheries. National Environmental Management: Waste Act (59/2008): Regulations regarding extended producer responsibility. Transport of Dangerous Goods: National Road Traffic Act, 1996 (Act No. 93 of 1996). SANS 10228:2022- The identification and classification of dangerous goods for transport by road and rail modes. |

- Other information

| Date of issue | 13 September 2023 |
| --- | --- |
| Date of previous issue | December 2014 |
| Version | 2 |
| Prepared by | Hygrotech Sustainable Solutions |

Notice to the reader

The information supplied in this Safety Data Sheet is correct to the best of our knowledge and belief at the time and date of its publication. Al reasonable effort was done to ensure that the data used is current and reliable. Neither the above-named supplier nor any of its subsidiaries assume any liability whatsoever for the accuracy or completeness of the information contained herein.

The information is supplied has been developed only as guidance for the safe handling, use, processing, storage, transportation, and disposal of the product and is not considered a warranty or quality specification.  Final determination of suitability of any material is the sole responsibility of the user.  All materials may present unknown hazards and should be used with caution. Although certain hazards are described herein, we cannot guarantee that these are the only hazards that exist.

END OF SAFETY DATA SHEET  Updated and revised: November 2025