Safety Data Sheet

Revision Date: 2022-10-24

- Product and company identification

Product Name: SUREBUFF

Identification: Agricultural Buffer

Recommended use: Buffer, only use as directed

Details of Supplier:

Hygrotech Properties (Pty) Ltd, 1 Gerard Braak Street, Pyramid, Pretoria, 0120

Emergency Telephone numbers:

Griffon Poison Information Centre: 082 446 8946

Hygrotech Head Office: 012 545 8000 (o/h)

- Hazard(s) identification

Classification of the substance or mixture

This product is classified as hazardous according to the criteria in South Africa - GHS classification and labelling of chemicals – SANS10234 and the Regulations for Hazardous Chemical Agents - 2021.

GHS Classification

| Hazard Class | Category | Hazard Statement Number |
| --- | --- | --- |
| Skin Corrosion/Irritation | 2 | H315 |
| Serious Eye Damage/Irritation | 2 | H319 |

Label Elements

Pictograms:

Signal Word:

Warning

Hazard Statements:

| Statement Number | Hazard Statement |
| --- | --- |
| H315 | Causes skin irritation. |
| H319 | Causes serious eye irritation. |

Precautionary Statements:

General -

| Statement Number | Precautionary Statement |
| --- | --- |
| P101 | If medical advice is needed, have product container or label at hand. |
| P102 | Keep out of reach of children. |
| P103 | Read carefully and follow all instruction. |

Prevention -

| Statement Number | Precautionary Statement |
| --- | --- |
| P280 | Wear protective clothing. |
| P264 + P265 | Wash hands and face thoroughly after handling. |

Response -

| Statement Number | Precautionary Statement |
| --- | --- |
| P302 + P352 | IF ON SKIN: Wash with plenty of water under the safety shower. |
| P332 + P317 | If skin irritation occurs: Get medical help. |
| P337 + P317 | If eye irritation persists: Get medical help. |
| P362 + P364 | Take off contaminated clothing and wash it before re-use. |
| P305 + P351 + P338 | IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing. |

Other Hazards

None known.

- Composition/information on ingredients

Substance/mixture

Mixture.

Ingredients with Hazard Concerns (GHS)

According to UN GHS criteria.

| Hazardous Component – Chemical Name | CAS Number | Weight - % | GHS Classification |
| --- | --- | --- | --- |
| Acetic acid | 64-19-7 | 10 – 25% | Flammable Liquids, Category 3. Skin Corrosion/Irritation, Category 2. Serious Eye Damage/Irritation Category 2. |
| Ammonium acetate | 631-61-8 | <10% | Not GHS classified. |

NOTE: The other ingredients do not cause or contribute toward the correct GHS classification of Surebuff and are therefore, in terms of the South African Regulations for Hazardous Chemical Agents - 2021; Regulation 14(b), not listed in the table above.

- First Aid measures

Description of First Aid measures

| General advice | In case of treatment resulting from excessive exposure, provide this SDS and product label to medical personnel. Emergency personnel should wear protective clothing appropriate to the type and degree of contamination. First Aid personnel should pay attention to their own safety. Immediately remove contaminated clothing and move the affected person away from the contamination area. Keep the person warm, calm, and covered up. |
| --- | --- |
| Inhalation | Allow the patient to rest in a well-ventilated area. If breathing is difficult, have qualified personnel administer oxygen. If breathing has stopped, use artificial respiration to support vital functions. Remove and isolate contaminated clothing and shoes. Make sure medical personnel are aware of the substance involved and take precaution to protect them. Obtain medical attention if concerned or unwell. |
| Ingestion | If this product is swallowed, call a poison control centre or medical practitioner for treatment advice. If conscious, rinse mouth thoroughly with water. Never give anything by mouth to an unconscious or convulsing person. Do not induce vomiting unless directed to do so by a medical professional. Have the patient lean forward with head down to avoid breathing in of vomits if spontaneous vomiting occurs. |
| Skin contact | Remove all contaminated clothing and shoes. Decontaminate the affected skin area with running water under the safety shower. Minimum recommended flushing time is 15 – 20 minutes, especially if an adverse skin reaction occurs. Obtain medical attention if irritation occurs or persist. Contaminated clothing should be washed prior to re-use. |
| Eye contact | IMMEDIATELY rinse/flush the eyes gently with running water for at least 15 minutes keeping the eyelids apart. Cold water must be used. Check for and remove contact lenses if easy to do so. Continue with the rinsing. Prevent rubbing of the eyes. If irritation occurs or persists, obtain medical attention. |

Most important symptoms/effects, acute and delayed

Skin and serious eye irritation.

Eyes: There may be irritation, swelling, pain and redness at the site of contact. Eye may water profusely.

Skin: Symptoms could include localized redness, blistering, itching and pain.

May cause respiratory irritation if inhaled.

Nausea, vomiting and diarrhoea if ingested.

Chronic symptoms may include skin cracking, coughing and inflammation of the eye tissue.

Indication of any immediate medical attention and special treatment needed

Notes to physician:

No specific antidote required. Treat patient symptomatically and supportively.

- Fire-fighting measures

| Extinguishing media | Small fires – use water spray, carbon dioxide, foam, or dry chemicals. Larger fires – use foam, water fog or water spray. Fire water must be contained for safe disposal. Do not use high volume water jets due to potential contamination. |
| --- | --- |
| Specific hazards arising from the chemical including thermal decomposition products | Product is non-flammable. When involved in a fire and exposed to extremely high temperatures, the components of this product may decompose to produce irritating vapours and hazardous compounds including carbon oxides (CO and CO2) and ammonia. |
| Special protective equipment | Fire responders should wear emergency equipment including positive pressure self-contained breathing apparatus with a full-face mask. If possible, prevent run-off water from entering storm drain bodies of water or other environmentally sensitive areas. |
| Additional measures | Fight the fire from a maximum distance and keep containers cool by spraying with water. Act in accordance with the site’s Internal Emergency Plan and the procedures for actions to be taken during and after an accident or other emergencies. |

- Accidental release measures

| Personal precautions, protective equipment, and emergency procedures | In case of a release, clear the affected area and protect people. Appropriately trained personnel dressed in the prescribed personal protective equipment should respond to uncontrolled releases. Emergencies should be handled in accordance with the facility’s pre-planned procedures. Contain the spill if it can be done without risk and clean-up immediately. Ventilate the area of the spill or leak, especially when in confined areas and do not breathe in fumes/vapours. Avoid contact of spilt material with eyes, skin and clothes and do not touch or walk through spilled material. Wear appropriate protective clothing recommended in Section 8 of the SDS. |
| --- | --- |
| Environmental precaution | Prevent spillage or further leakage (if safe to so) and the contamination of environmentally sensitive areas. Do not allow the spilt product to enter water courses and drains and avoid contact with soil. Report spills and releases as required to appropriate authorities if the spilt product has caused environmental pollution (sewers, water ways, or soil). |
| Containment and clean-up | For small spills – clean up using gloves, safety glasses and appropriate body protection. Use absorbent material from the spill kit to soak up the spill. Sweep up and place into a container for disposal to an approved landfill. For large spills - personal protective equipment should be level C: triple gloves, chemical resistant suit, and air purifying respirator with a high-efficiency particulate filter. Contain the spill using absorbent cushions/socks/pads in the spill kit. Collect the spilt product in suitable containers for proper disposal. If necessary, dike the spill to prevent release from contaminating environmentally sensitive areas. |
| Reference to other SDS sections | See Section 1 for emergency contact information. See Section 8 for information on appropriate personal protective equipment. See Section 13 for additional waste treatment information. |

- Handling and storage

| Precautions for safe handling | Wear the protective clothing as indicated in Section 8 of this SDS. Prevent personal exposure to the product – prevent skin and eye contact. Wash hands and face thoroughly after handling the product and do not eat, drink, smoke or apply cosmetics while handling. Always use in a well- ventilated work area. Open containers slowly on a stable surface. Release any pressure build-up from the container before totally opening the lid. Locate emergency showers and eye-rinsing facility near the work/handling area. Maintain good normal industrial hygiene and housekeeping practices in areas where the product is used/handled. Remove contaminated clothing immediately if the product gets inside. Contaminated work clothing should not be allowed out of the workplace. Regular cleaning of work area and work clothing is recommended. Keep unprotected persons away from the area where the product is being applied. |
| --- | --- |
| Conditions for safe storage, including any incompatibilities | The entrance to storage facilities should be granted only to appropriately trained personnel. Keep out of reach of children, uninformed persons, and animals. Containers must always be clearly labelled and tightly closed when not in use. Store containers in a cool dry location, away from direct sunlight and sources of intense heat, or where freezing is possible. Store away from incompatible materials (see Section 10). Inspect all incoming containers before storage to ensure that containers are properly labelled and are not damaged. Empty containers should be handled with care. It is recommended to have appropriate spill control kits equipped with clean-up tools near storage areas (see Section 6). Store in accordance with national and local regulations. |

- Exposure controls/personal protection

## Components with workplace control parameters – National Occupational Exposure Limits

## An occupational exposure limit for Acetic acid has established by the South African Department of Labour and Employment.

| Component | Type | Control Parameter | Update | Basis |
| --- | --- | --- | --- | --- |
| Acetic acid | OEL- eight- hour TWA | 20 ppm | 2021 | South African RELs* |
|  | OEL- STEL/C | 30 ppm | 2021 | South African RELs* |

| *REL: | Recommended Exposure Limit. |
| --- | --- |
| OEL- eight-hour TWA: | Occupational Exposure Limit- Time Weighted Average. Calculated over an eight-hour working day, for a five-day working week. |
| ppm: | Parts per million |

| Appropriate Engineering controls | Use with adequate general or mechanical exhaust ventilation to prevent inhalation of spray or mist and to maintain airborne concentration below the occupational exposure limit. Eyewash stations and safety showers should be near areas where this product is used or sprayed. |
| --- | --- |
| Individual protection measures | Hand: impervious chemical resistant gloves recommended for hand protection (e.g., butyl rubber, nitrile rubber, etc.). The gloves should be replaced immediately in case of damage or signs of wear. Eye/Face: safety eyewear compliant with an approved standard should be used when a risk assessment indicates this is necessary to avoid eye exposure to liquids and splashes. Safety goggles together with a face shield is recommended when a respirator is not used. Skin: impervious overalls, apron, shoes, and socks as required to prevent skin contact and contamination of personal clothing. Overalls must be buttoned to the neck and sleeves worn over the gloves. Respiratory: protection selection must be based on known or anticipated exposure levels, the hazards of the product and the safe working limits of the selected respiratory equipment. In operations where exposure levels are exceeded or expected to be high, an approved respirator (full face mask) with a particulate filter and an organic vapour cartridge or supplied air respirator should be used. Respirator selection and use should be based on contaminant type, form, and concentration. For emergency conditions, use an approved positive-pressure self-contained breathing apparatus. |
| Environmental exposure controls | In accordance with the national and local legislation for the protection of the environment, it is recommended to avoid environmental spillage or releases of both the product and its container. |

- Physical and chemical properties

| Phase | Liquid |
| --- | --- |
| Colour | Light blue |
| Odour | Slight acetic acid odour |
| pH (1%) | 4.5 - 5 |
| Density | 1.1 g/cm3 |
| Solubility | Soluble in water |
| Octanol/water partition coefficient: | Not determined |
| Flash Point (°C) | >90 |
| Flammability | Not flammable |

Other Hazard Information

None.

- Stability and reactivity

| Reactivity: | The product is not reactive under normal ambient and anticipated storage and handling conditions of temperature and pressure. |
| --- | --- |
| Chemical stability: | Hazardous polymerization will not occur. Stable under normal ambient conditions of use, storage, and transport. |
| Conditions to avoid: | Storage without good ventilation and exposing to excessive heat. |
| Incompatible materials: | Strong oxidizers and strong bases. |
| Hazardous decomposition products: | Does not decompose when used for intended uses. Could decompose under fire or during burning, and at high temperatures releasing oxides of carbon (CO and CO2) and ammonia. |

- Toxicological information

Information on likely routes of exposure

The product can be absorbed into the body by inhalation of its aerosol and by skin/eye contact.

Information on toxicological effects

Surebuff is of low acute toxicity.

Assessment of acute toxicity:

Experimental toxicological test data is not available for the product. Assessment of toxicological effects is based on calculation and ingredient data.

Assessment of acute toxicity is based on Acetic acid.

| Product/ingredient Name | Dose Acute - | Species | Test Result |
| --- | --- | --- | --- |
| Acetic acid | 3 310 mg/kg | Rat | LD50 Oral |
| Acetic acid | 8.5 mg/L air | Rat | LC50 Inhalation – 4h |

| Irritation Dermal/Skin and Eyes: | Assessment of irritation effects (skin/eyes): Based on available data, the classification criteria are met for skin and serious eye irritation. Acetic acid - >10% acetic acid was shown to be irritating to the rabbit eye. |
| --- | --- |
| Respiratory/Skin Sensitization: | Assessment of sensitization: Based on available data, the classification criteria are not met. |
| Germ Cell Mutagenicity: | Assessment of mutagenicity: Based on available data, the classification criteria are not met. |
| Carcinogenicity: | Assessment of carcinogenicity: Based on available data, the classification criteria are not met. |
| Reproductive Toxicity and Developmental Toxicity: | Assessment of reproduction toxicity: Based on available data, the classification criteria are not met. |
| Specific Target Organ Toxicity (Single Exposure): | Assessment of STOT (SE): Based on available data, the classification criteria are not met. |
| Specific Target Organ Toxicity (Repeated Exposure): | Assessment of STOT (RE): Based on available data, the classification criteria are not met. |
| Aspiration Hazard: | Assessment of Aspiration Hazards: Based on available data, the classification criteria are not met. |

Delayed and immediate effects as well as chronic effects from short and long-term exposure

The delayed, immediate, and chronic effects of the product are discussed in Section 4 of the SDS.

- Ecological information

Ecotoxicity

The product is of low toxicity to aquatic systems.

The product will not enter the environment under normal use if used as indicated on the label. Care should however be taken to avoid any additional release, for example through inappropriate disposal.

No eco-toxicological data is available for the formulated product. This ecological assessment is based on data available for acetic acid.

| Ingredient | Species and Genus | Exposure (hours/days) | Result in fresh water |
| --- | --- | --- | --- |
| Acetic acid | Crustacea (Daphnia magna) | 48h | Acute EC50: >1000mg/L |
| Acetic acid | Fish (Oncorhynchus mykiss) | 96h | Acute EC50: >1 000mg/L |

Other Environmental and Adverse Effects:

| Environmental effect | Environmental Effect Applicable to Ingredient | Description |
| --- | --- | --- |
| Persistence and degradability: | Acetic acid | Using a modified Organization of Economic Cooperation and Development (OECD) protocol, 75% degradation was reported in 14 days using garden soil as an inoculum. This indicates that biodegradation is an important environmental fate process in soil. |
| Bioaccumulative potential: | Acetic acid | An estimated BCF of 3 suggests the potential for bioconcentration in aquatic organisms is low. |
| Mobility in soil: | Acetic acid | An experimentally derived Koc value of 1.0 indicates that acetic acid is expected to have very high mobility in soil. Acetic acid is expected to volatilize from dry soil surfaces based upon its vapor pressure. If released into water, acetic acid is not expected to adsorb to suspended solids and sediment based upon the Koc value. |
| Other adverse effects: | Acetic acid | None known. |

- Disposal information

| Waste handling and disposal methods: | Rinse empty container 3 times with a volume of water equal to one tenth of the volume of the container and add to the spray tank. Avoid and minimize the generation of waste. Dispose product related waste in accordance with all local regulations and prevent the contamination of water, food, or feed by storage or disposal of the waste. Waste product or empty containers must not be disposed of as part of general waste. |
| --- | --- |
| Disposal of packaging: | The product and its container must always be disposed of in a safe manner. Empty containers and offer for recycling if an available option. Recondition if appropriate, or puncture/flatten and dispose of in a hazardous waste landfill, or by other procedures approved by the local authorities. |

- Transport information

The product is not classified for the transport of Dangerous Goods purposes.

- Regulatory information

Safety, health, and environmental regulations specific for the product in question

| Hazard symbol |  |
| --- | --- |
| Classification: | Xi – Irritant. |
| Risk phrases: | R36: Irritating to eyes. R38: Irritating to skin. |

Key to Abbreviations

| AND | European Provisions concerning the International Carraige od Dangerous Goods by inland Waterways |
| --- | --- |
| ADR | The European Agreement concerning the International Carraige of Dangerous Goods by Road |
| ATE | Acute Toxicity Estimate |
| CAS Number | Chemical Abstracts Service Number |
| COD | Chemical Oxygen Demand |
| GHS | Globally Harmonised System of Classification and Labelling of Chemicals |
| IATA | International Air Transport Association |
| ICAO | International Civil Aviation Organisation |
| IMDG | International Maritime Dangerous Goods |
| LogPow | Logarithm of the octanol/water partition coefficient |
| LD50 | Lethal Dose 50 |
| LC50 | Lethal Concentration 50 |
| RID | The Regulations concerning the International Carraige of Dangerous Goods by Rail |
| SDS | Safety Data Sheet |
| STOT | SpecificTarget Organ Toxicity |
| TWA | Time Weighted Average |
| UN | United Nations |

No known specific country national and/or local regulations applicable to the product (including its ingredients). A summary of country specific general laws/regulations are supplied below.

Country Specific Legal Requirements

| Country | Requirements |
| --- | --- |
| South Africa: | Registration Requirements: Fertilizer, Farm Feeds, Agricultural Remedies and Stock Remedies Act, 1947 (Act 36 of 1947). Pesticide Handling, Storage and Disposal Safety: SANS10206: 2020. Safety Data Sheet and Occupational Exposure Limit Requirements: Regulations for Hazardous Chemical Agents – 2021 – SA Occupational Health and Safety Act. SANS11014:2010. Control of and handling of poisonous/hazardous and non-poisonous/non-hazardous substances/chemicals in workplaces: Hazardous Substances Act, 1973 (Act No.15 of 1973). Occupational Health and Safety Act No. 85 of 1993. |

- Other information

| Date of issue: | 24 October 2022 |
| --- | --- |
| Date of previous issue: | June 2017 |
| Version: | 2 |
| Prepared by: | Hygrotech Sustainable Solutions |

Notice to the reader

The information supplied in this Safety Data Sheet is correct to the best of our knowledge and belief at the time and date of its publication. Al reasonable effort was done to ensure that the data used is current and reliable. Neither the above-named supplier nor any of its subsidiaries assume any liability whatsoever for the accuracy or completeness of the information contained herein.

The information is supplied has been developed only as guidance for the safe handling, use, processing, storage, transportation, and disposal of the product and is not considered a warranty or quality specification.  Final determination of suitability of any material is the sole responsibility of the user. All materials may present unknown hazards and should be used with caution. Although certain hazards are described herein, we cannot guarantee that these are the only hazards that exist.

END OF SAFETY DATA SHEET